CNC PCB PLOTTER

Design, Firmware & Cloud-Integrated Motion Control

Micro-Controller Lab : PIC 16F877A

Charbel Estephan 20231222 
Habib Bou Maroun 20223435

# Abstract

This report documents how to designed and built a CNC PCB plotter from scratch. The machine takes a Gerber file (the standard format that PCB manufacturers use) and draws the copper traces directly onto a blank copper board using a pen. The whole thing runs autonomously once you upload the file from a browser.

The system has three main parts talking to each other: A Python backend running on Google Cloud that converts the Gerber file into machine specific instruction set, an ESP32 that handles WiFi and the webpage, and a PIC16F877A microcontroller written in assembly that actually drives the stepper motors. The PIC is doing all the real-time work from running a Bresenham line algorithm in 24-bit integer arithmetic to generating step pulses at up to 1000 Hz, and maintaining the coordinate system. The motors run at 1/16 micro-stepping through A4988 driver modules, giving a resolution of 400 steps/mm on both axes.

# 1. Introduction

## 1.1 Why Build This

Getting a PCB made professionally takes at least a week and only makes sense if you order several at a time. On the other hand, doing toner transfer at home is messy. A plotter seemed like the right middle ground: it draws the resist mask directly onto the board under numeric control.

It is also a genuinely interesting engineering problem. To make it work you need motion control, embedded firmware, wireless communication, cloud deployment, and mechanical design all working together. Every single part of the stack is under direct control, which made debugging painful at times but also made it very educational.

## 1.2 Goals

A few concrete targets before starting:

- The machine should accept Gerber files because every PCB CAD tool exports Gerber.

- The working area should be at least 250 mm × 220 mm, large enough for most single-sided hobby boards.

- It should be controllable from a browser. No installing software, no USB cable required to start a job.

- Gerber parsing should not happen on the microcontroller. That keeps the embedded firmware small and lets me fix parser bugs without reflashing hardware.

# 2. System Architecture

The plotter is split into three layers, each running on a different processor. The split is not arbitrary, it follows the natural boundary between what needs to be fast, what needs to be smart, and what needs to be heavy.

## 2.1 The Three Layers

### Layer 1 — Cloud Backend (Python / Flask / Google Cloud Run)

Gerber parsing lives here. The backend is a Flask app running in a Docker container on Cloud Run. When the ESP32 uploads a Gerber file, the backend parses it, expands pad geometry into filled pen strokes, clamps everything to the machine's working envelope, and sends back a stream of 14-byte motion commands.

Why Cloud Parsing? 
Gerber files can be hundreds of kilobytes, and building a robust Gerber parser in C on a memory-constrained embedded processor is a terrible idea. Python's regex handles the file in milliseconds no hardware involved.

### Layer 2 — Communication Hub (ESP32 / Arduino C++)

The ESP32 is the network interface. It hosts the web page, uploads Gerber to the cloud over HTTPS, fetches the motion commands back in chunks, and relays them one at a time to the PIC over a UART link. In essence it is a protocol translator: WiFi/TLS on one side, simple 9600-baud UART on the other.

### Layer 3 — Motion Controller (PIC16F877A / MPASM Assembly)

The PIC does the real-time heavy lifting. It receives ASCII motion frames, converts them to step counts, runs the Bresenham algorithm to generate coordinated XY pulses, drives the stepper driver modules, polls the limit switches during homing, and updates the LCD. Everything is written in MPASM assembly, because the PIC at 4 MHz has tight timing constraints and the arithmetic needs to be verified at the instruction level when things go wrong.

| Layer | Processor | Language | Main job |
| --- | --- | --- | --- |
| 1 - Backend | Cloud Run | Python / Flask | Gerber parsing, coordinate generation |
| 2 - Hub | ESP32 | Arduino C++ | WiFi, TLS, UART bridge |
| 3 - Motion | PIC16F877A @ 4 MHz | MPASM Assembly | Bresenham, step pulses, LCD display |

## 2.2 How a Plot Job Flows

Here is what actually happens end-to-end when you press 'Start' in the browser:

- The user selects a Gerber file in the ESP32's web interface.

- The ESP32 opens a TLS connection to Cloud Run and streams the file as a chunked HTTP upload.

- Cloud Run parses the file, expands all pad geometry, clamps coordinates to 250×220 mm, and writes the result to a temp file. It returns a JSON ticket: {job_id, total_cmds, total_bytes}.

- The ESP32 then fetches the command stream in 4 kB slices via GET /fetch?job=…&offset=…

- Each 14-byte frame is forwarded to the PIC over UART2. The ESP32 waits for ACK before sending the next one.

- After the last frame, the ESP32 sends DELETE /job/<id> to clean up the temp file on the server.

# 3. Hardware Components

## 3.1 PIC16F877A — Motion Controller

The PIC16F877A is the motion controller. It drives the steppers, reads limit switches, handles UART communication with the ESP32, and runs the LCD. It is an 8-bit Harvard RISC processor running at 4 MHz deliberately underpowered by modern standards, which forces you to be intentional about every instruction you write.

## 3.2 ESP32 — Communication Hub

The ESP32 DevKit v1 handles all the networking. It runs at 240 MHz with 520 kB SRAM, supports 802.11 b/g/n WiFi natively, and has a proper TLS stack (Mbed TLS) baked into the SDK. It also has UART2 for the PIC link.

## 3.3 NEMA 17 17HS3401S — Stepper Motors

Both axes use a NEMA 17 17HS3401S bipolar stepper. This is effectively the standard motor for home-scale CNC and 3D printing (it is physically large enough to move a pen carriage smoothly, and supported by every driver, coupler, and mechanical mount on the market). The 17HS3401S specifically is a short-body, low-current variant which keeps heat manageable when the driver is set to around 1.3 A per phase.

| Parameter | Value |
| --- | --- |
| Step angle | 1.8° (200 full steps/revolution) |
| Phase current (rated) | 1.3 A |
| Holding torque | 28 N·cm |
| Shaft | T8- screw |
| Body length | 34 mm |

The T8 is an 8 mm diameter trapezoidal lead screw with a 4-start thread giving 8 mm of linear travel per revolution. Combined with the 200-step motor and 1/16 micro-stepping: (200 × 16) ÷ 8 = 400 steps per millimeter. That is 2.5 µm per micro-step on paper about 160× finer than the pen tip, so the electronics are never the limiting factor on print quality.

# 4. The Stepper Motor Driver — How It Actually Works

This section goes into more depth than the rest of the report because the stepper driver is the most misunderstood part of the system. Most descriptions of CNC projects say 'connect STEP and DIR to the driver and it works' which is true, but gives you no understanding of what the driver is doing or why the motor behaves the way it does. This section explains the full picture.

## 4.1 The Problem the Driver Solves

Stepper motors are driven by current, not voltage. The torque a stepper produces is proportional to the current flowing through its coils not to the voltage applied. If you just connect a 12V supply directly to the motor windings, the current would ramp up at a rate of V/L (voltage divided by winding inductance, typically 2–5 mH), quickly exceed the rated current, and saturate the iron core, destroying the motor or the driver or both.

The other problem is that generating micro-stepping waveforms in software on the PIC is not feasible. At 1/16 micro-stepping and 1000 Hz step rate, the PIC would need to switch four H-bridge transistors, regulate current on two coils simultaneously, and compute the correct sinusoidal current level — all at 16,000 times per second. At 4 MHz that is 250 instruction cycles per step. There is no room left for Bresenham, UART, or the LCD.

A dedicated driver IC solves both problems. It accepts simple STEP and DIR pulses from the PIC, handles all H-bridge switching and current regulation internally, and operates its power stage from 12V while presenting a 3–5V logic interface. The PIC just counts steps.

|  | The fundamental role of the driver: translate digital step pulses from a microcontroller running at logic levels into precisely regulated current waveforms in the motor coils at power levels the microcontroller could never handle directly. |
| --- | --- |

## 4.2 Inside the A4988

The A4988 from Allegro Microsystems is a DMOS microstepping driver with built-in translator logic. It is the most commonly used stepper driver in desktop CNC and 3D printing equipment. Understanding it requires understanding five things that happen inside the chip simultaneously.

### 4.2.1 The Translator Logic

The translator is the block that takes STEP pulses and converts them into the correct H-bridge switching pattern. Every rising edge on the STEP pin advances an internal state machine by one position. The state machine drives two full H-bridges — one per motor phase — according to a predetermined sequence.

Without micro-stepping, the sequence follows the classic four-phase pattern (A+B+, A-B+, A-B-, A+B-). With 1/16 micro-stepping enabled, the translator instead steps through a 32-entry lookup table on each STEP pulse, outputting a different target current level for each phase at every position. This is completely invisible to the PIC — it just keeps pulsing STEP and the translator handles the rest.

### 4.2.2 The PWM Chopper — Current Regulation

The lookup table outputs a target current level, but something has to actually enforce that current in the real motor coil. That is what the PWM chopper does. It runs a continuous feedback loop at approximately 20–40 kHz:

- The H-bridge applies 12V across the motor coil. Current ramps up at a rate of V/L (roughly 2–4 A/ms for the 17HS3401S).

- A small sense resistor (R_sense, soldered on the breakout board) is in series with the coil. The voltage across it equals I × R_sense and is read by an internal comparator.

- When the sensed current reaches the target level from the lookup table, the comparator fires and the PWM latch cuts off the H-bridge.

- After a fixed off-time (set by external components, typically 20–40 µs), the bridge re-enables and the cycle repeats.

The net result is a sawtooth current waveform whose average value equals the target level from the table. The 12V supply charges the coil quickly and the chopper regulates the average — this is why the motor runs from 12V even though its rated voltage is only 2.8V. Higher supply voltage means faster current rise, which means more torque at speed.

|  | The fixed off-time chopper is the reason CNC motors run from a much higher voltage than their rated spec. The driver is not applying 12V DC to a 2.8V motor — it is using 12V to charge the coil inductance quickly, then chopping the current back to the rated level. The result is better high-speed torque compared to running at 2.8V directly. |
| --- | --- |

### 4.2.4 The H-Bridge — How Current Direction Works

A bipolar stepper needs current to flow in both directions through each winding to complete full rotation. A single transistor cannot do this — you need an H-bridge, which is four switching elements arranged so current can flow either way:

+12V

|

[FET1]   [FET3]   ← high-side (P-channel DMOS)

|       |

+---M---+      ← motor winding between midpoints

|       |

[FET2]   [FET4]   ← low-side (N-channel DMOS)

|

GND / R_sense

Forward current:  FET1 + FET4 ON,  FET2 + FET3 OFF

Reverse current:  FET3 + FET2 ON,  FET1 + FET4 OFF

The A4988 contains two complete H-bridges — one for Phase A (coils 1A/1B) and one for Phase B (coils 2A/2B). The translator logic switches these bridges in the correct sequence as STEP pulses arrive. The PIC never needs to think about any of this.

## 4.3 Setting the Current — VREF Explained

This is the most important calibration step and the one most likely to go wrong if you do not understand what you are doing. The maximum phase current is set by a trim potentiometer on the breakout board, which sets a reference voltage on the VREF pin of the IC. The relationship is:

A4988 formula:   I_max = VREF / (8 × R_sense)

A4988 module (R_sense = 0.068 Ω), target I = 1.1 A:

VREF = 1.1 × 8 × 0.068 = 0.598 V  → set to ~0.6 V

To calibrate: power the board without the motor connected, touch a multimeter positive probe to the VREF test point (centre of the trim pot), negative to GND, and adjust until you reach the target voltage. Do not skip this step — VREF too high will cook the driver; too low and the motor misses steps under load.

|  | Important: always attach the small aluminium heatsink to the top of the IC before running at currents above about 0.5 A. At 1.1 A per phase the driver dissipates roughly 1.5–2 W. Without a heatsink the thermal protection trips at ~150°C junction temperature, causing random shutdown that looks like firmware bugs. |
| --- | --- |

## 4.4 Micro-Stepping Mode Selection

The micro-stepping resolution is set by three logic pins: MS1, MS2, MS3. On the breakout board, pull-up resistors hold all three HIGH by default (1/16 mode). The truth table is:

| MS1 | MS2 | MS3 | Mode | Steps/rev | Steps/mm on T8 |
| --- | --- | --- | --- | --- | --- |
| LOW | LOW | LOW | Full step | 200 | 25 |
| HIGH | LOW | LOW | Half step | 400 | 50 |
| LOW | HIGH | LOW | Quarter step | 800 | 100 |
| HIGH | HIGH | LOW | Eighth step | 1,600 | 200 |
| HIGH | HIGH | HIGH | Sixteenth step | 3,200 | 400 |

In this project all three MS pins are left floating to their board pull-ups (all HIGH), so the driver is always in 1/16 mode. No wiring from the PIC to the MS pins is needed. The PIC firmware assumes 400 steps/mm at all times.

## 4.5 A4988 vs DRV8825 — Which One and When

I started the project with the A4988 during full-step testing and switched to the DRV8825 when moving to 1/16 micro-stepping. Both modules are pin-compatible so swapping them required zero firmware changes. The practical differences that affected this project:

| Feature | A4988 | DRV8825 | Notes |
| --- | --- | --- | --- |
| Max phase current | 2.0 A | 2.5 A | Both fine at 1.1 A — DRV8825 has more headroom |
| Max micro-stepping | 1/16 | 1/32 | DRV8825 is a future upgrade path |
| Step pulse minimum | 1 µs | 1.9 µs | PIC generates ~4 µs pulses — both satisfied |
| Current formula | VREF/8Rs | VREF/2Rs | Must not confuse these when setting VREF |
| Mid-band noise | More | Less | DRV8825 runs noticeably quieter at 1/16 |
| Cost | Cheaper | Slightly more | Small difference at these quantities |

## 4.6 Wiring the Driver to the PIC

The interface between PIC and driver uses four wires: two per axis (STEP and DIR). Both the A4988 and DRV8825 accept 3–5.5V / 2.5–5.25V logic, so the PIC's 5V outputs connect directly with no level shifting needed.

| Driver Pin | PIC Pin | Signal | Notes |
| --- | --- | --- | --- |
| STEP (X axis) | PORTB,0 | X_STEP | Each rising edge = one micro-step |
| DIR (X axis) | PORTB,1 | X_DIR | Set before first STEP pulse; hold for entire move |
| STEP (Y axis) | PORTB,2 | Y_STEP | Same as X |
| DIR (Y axis) | PORTB,3 | Y_DIR | Same as X |
| ENABLE | GND | ~ENA | Tie permanently LOW — always enabled |
| RESET | 5V | ~RST | Tie permanently HIGH — never reset mid-job |
| MS1/MS2/MS3 | (pull-up) | Mode | Left to board pull-ups → 1/16 mode, no PIC wiring needed |
| VMOT | 12V PSU | Power | 100 µF 35V electrolytic cap to GND required nearby |
| 1A, 1B | Motor coil A | Phase A | Polarity sets forward direction |
| 2A, 2B | Motor coil B | Phase B | Polarity sets forward direction |

The 100 µF decoupling capacitor on VMOT is not optional. The chopper generates sharp current spikes each switching cycle; without the capacitor these produce voltage spikes on the supply rail that can exceed the driver's absolute maximum rating and destroy it. I learned this the hard way on the first driver module.

### Step Pulse Generation in Firmware

On the PIC side, each step is generated by a brief BSF/BCF pair on the PORTB pin:

; Generate one step pulse on the X axis

BSF   PORTB, 0     ; STEP → HIGH  (rising edge triggers step in driver)

NOP                ; short hold — A4988 needs ≥1 µs, DRV8825 ≥1.9 µs

NOP                ; at 4 MHz each NOP = 250 ns; four NOPs = 1 µs minimum

NOP

NOP

BCF   PORTB, 0     ; STEP → LOW   (ready for next pulse)

; After this, STEP_DELAY loop runs to set the inter-step gap (feed rate)

The DIR pin must be stable before the first STEP pulse. In the Bresenham implementation, XDIR_F and YDIR_F flags are set based on an explicit compare of target vs. current position before entering the motion loop — so the direction is locked in before any pulses are generated.

## 4.7 Why 1/16 Micro-Stepping Made Such a Difference

The upgrade from full-step to 1/16 micro-stepping was the single most impactful change in the entire project in terms of visible output quality. The differences were immediate and obvious:

- Diagonal lines: at full-step you can see a faint staircase pattern on 45-degree traces. At 1/16 they look perfectly straight.

- Noise: the motor went from a loud resonant whine to a soft hiss. The resonance frequency at full-step falls right in the audible range at typical CNC feed rates; micro-stepping shifts it high enough that the motor passes through it too quickly to excite it.

- Vibration: at full-step you can feel the table vibrating during a move. At 1/16 it is essentially smooth. This matters for line quality because vibration means pen wobble.

The cost is that the PIC has to generate 16× more step pulses for the same linear travel. At 40 mm/s and 400 steps/mm, that is 16,000 pulses per second. At 4 MHz, one pulse every 250 instruction cycles is tight but workable — the Bresenham inner loop plus step pulse plus minimum delay fits in that window with a small margin.

# 5. Firmware Architecture

## 5.1 PIC16F877A — MPASM Assembly

### 5.1.1 Main Loop

The PIC firmware is a single-threaded assembly program. After initialisation and homing it drops into a tight loop that repeats for every motion command:

MAIN

CALL    RECEIVE_LINE       ; wait for a 14-byte ASCII frame on UART

BTFSC   ENDFLAG, 0         ; is it the 'END' marker?

GOTO    PLOT_DONE          ; yes → stop and print PLOT COMPLETE

CALL    PARSE_AND_DISPLAY  ; unpack X, Y, pen; update LCD

CALL    EXECUTE_MOTION     ; Bresenham + step pulses

CALL    SEND_ACK           ; send 'A' back to ESP32

GOTO    MAIN

### 5.1.2 Peripheral Init (what each register actually does)

- TRISC = 0x81: makes RC7 (UART RX) an input and RC6 (TX) an output. The asymmetric pattern is because the USART overrides the TX TRIS bit automatically, but not RX.

- TXSTA = 0b00100100: selects asynchronous 8-bit mode, BRGH=1 for high-speed baud rate generation.

- SPBRG = 25: with BRGH=1 and Fosc=4 MHz, gives 4,000,000 ÷ (16×26) = 9,615 baud (+0.16% error — well within UART tolerance).

- RCSTA = 0b10010000: enables the serial port (SPEN) and the receiver (CREN).

- ADCON1 = 0x06: disables the ADC on all pins, reclaiming PORTA and PORTE as pure digital I/O.

- CMCON = 0x07: disables the comparators, which otherwise interfere with limit switch reads on PORTA.

- TRISA = 0b00001100: keeps PORTA,2 and PORTA,3 as inputs for the X and Y limit switches.

### 5.1.3 Memory Map (relevant registers)

| Address | Name | What it holds |
| --- | --- | --- |
| 0x30–0x3D | BUF[14] | The current incoming ASCII frame |
| 0x50–0x55 | XTGT / YTGT | Target position, 24-bit, 3 bytes each |
| 0x56–0x5B | XCUR / YCUR | Current machine position, 24-bit |
| 0x5C–0x64 | DX / DY / BERR | Bresenham deltas and error accumulator, 24-bit |
| 0x70–0x72 | MULX/MULH/MULL | Result of the step conversion multiply |
| 0x68–0x6A | XDIR_F / YDIR_F / DOM_FLAG | Direction and dominant-axis flags |

## 5.2 The 24-Bit Bresenham Algorithm

This is the core of the motion controller. Coordinated XY motion requires that both axes reach their targets at the same time, tracing a straight line between start and end points. Bresenham's algorithm does this using only integer addition and subtraction — no floating point, no trigonometry, which is essential on a processor with no hardware multiplier.

DX = |x_target - x_current|

DY = |y_target - y_current|

if DX >= DY:

dominant = X,  minor = Y

else:

dominant = Y,  minor = X

BERR = D_dominant / 2    ← starting error (biased to centre)

loop D_dominant times:

step dominant axis

BERR -= D_minor

if BERR < 0:

step minor axis

BERR += D_dominant

The 24-bit implementation is needed because step counts at 1/16 micro-stepping reach up to 100,000 (250 mm × 400 steps/mm), which overflows 16-bit integers. Every arithmetic operation on the three-byte values requires explicit carry propagation. The critical pattern:

; Add two 24-bit numbers stored as (HI:MD:LO) byte triples

ADD24:

MOVF    TMP3, W

ADDWF   RES_LO, F      ; add low bytes

BTFSC   STATUS, C      ; did it carry into middle?

INCF    RES_MD, F      ;   yes → bump middle byte

BTFSC   STATUS, Z      ; did that INCF roll over to zero?

INCF    RES_HI, F      ;   yes → bump high byte too

MOVF    TMPL, W

ADDWF   RES_MD, F

BTFSC   STATUS, C

INCF    RES_HI, F

MOVF    TMPH, W

ADDWF   RES_HI, F

RETURN

The double-check (BTFSC STATUS,C then BTFSC STATUS,Z) is the key insight. The first catches the carry from the ADDWF. The second catches the case where the INCF of the middle byte itself rolls from 0xFF to 0x00 — which sets the Zero flag, not the Carry flag. Missing this second check causes a carry loss bug that manifests as coordinates being exactly 65,536 steps short. I hit this bug and spent a long time finding it.

## 5.3 ESP32 State Machine

The ESP32 loops through states: IDLE → SENDING (upload to Cloud Run) → READING_JOB (parse ticket JSON) → FETCH_CONNECT / FETCH_BODY (get next chunk) → PLOT_CHUNK (forward frames to PIC) → WAITING_ACK (blocked on PIC response) → CLEANUP → DONE. Each TLS session is closed after one 4 kB chunk to avoid Cloud Run idle timeout.

## 5.4 Flask Backend

The backend is about 390 lines of Python. The three endpoints are POST /convert (parse Gerber, return job ticket), GET /fetch?job=&offset=&len= (serve a byte slice), and DELETE /job/<id> (clean up). The Gerber divisor is extracted at runtime from the %FSLAX/Y header: divisor = 10^(number of decimal digits). Final frame format: f"I{x:06d}{y:06d}{pen}" — 14 characters, always, asserted before return.

# 6. Communication Protocol

## 6.1 Frame Format

Every motion command is a 14-byte ASCII string terminated by a single LF:

I  XXXXXX  YYYYYY  Z  \n

I       = 1 byte  = start marker, always the literal character 'I'

XXXXXX  = 6 bytes = X position in steps, zero-padded ASCII decimal

YYYYYY  = 6 bytes = Y position in steps, zero-padded ASCII decimal

Z       = 1 byte  = pen state: '1' = down, '2' = up

\n      = 1 byte  = line feed terminator

End-of-stream: the literal string  END\n

Example: I0012000005001\n

X = 1200 steps = 3.00 mm,  Y = 500 steps = 1.25 mm,  pen down

## 6.2 Why ASCII and Not Binary

An early version of the protocol used binary encoding: one command byte, two 16-bit little-endian coordinate fields, one ACK byte. It was about half the size on the wire. I replaced it because it was genuinely unpleasant to debug — a single dropped byte on a binary stream corrupts every subsequent frame silently, and diagnosing that on a 9600-baud logic analyser trace is not fun.

ASCII with a fixed start marker ('I') and newline terminator is self-synchronising. The PIC can always find the next valid frame by scanning for 'I' followed by 12 digits and a valid pen byte. At 9600 baud and roughly one command per millimetre of travel, the extra bytes cost nothing measurable — the link never even approaches saturation.

## 6.3 Flow Control

Strict stop-and-wait: the ESP32 sends one frame, then waits for the PIC's two-byte response (pen-state echo + 'A' for ACK, or 'R' for NAK / retransmit). The important thing is that this is applied against a streaming data source — the ESP32 has the next frame ready to write the moment ACK arrives, so from the motor's perspective successive frames arrive without any meaningful gap. This is what prevents ink blobs at corners. More detail in Section 8.1.

# 7. Gerber Processing Pipeline

## 7.1 What Is a Gerber File

Gerber (RS-274X) is the format PCB manufacturers use. It is a text file describing shapes using a small command set: FS (coordinate format declaration), MO (units), AD (aperture definition), and operation codes D01/D02/D03 (draw/move/flash). The first thing the parser does is read the FS header to determine the coordinate divisor, which varies between files:

| FS header | Decimal digits | Divisor | Example: X100000 = |
| --- | --- | --- | --- |
| %FSLAX24Y24*% | 4 | 10,000 | 10.0 mm |
| %FSLAX36Y36*% | 6 | 1,000,000 | 0.1 mm |
| %FSLAX46Y46*% | 6 | 1,000,000 | 0.1 mm |

Getting this wrong was one of my early bugs — the parser was using a hardcoded divisor of 10,000 when most modern Gerbers need 1,000,000. The board was drawn at 1/100th scale. The fix was extracting the divisor from the header at runtime using a regex.

## 7.2 Aperture Expansion

When the Gerber file places a pad (D03 command), the parser does not just draw a dot — it expands the pad shape into a series of filled pen strokes using a 0.2 mm raster step (half the 0.4 mm pen width). Three shapes are handled:

- Circles: boundary stroke + horizontal raster + vertical raster. Both raster directions ensure complete coverage even with a worn pen tip.

- Rectangles: double boundary pass (to sharpen edges) + both-axis raster fill.

- Obrounds (stadium pads): boundary along the exact curved outline + serpentine fill within the stadium shape.

## 7.3 Coordinate Output

After all coordinates are collected: translate so the minimum corner sits at (0,0), mirror X if needed for copper-side-up drawing, add a 2 mm safety margin on both axes, then multiply by 400 to get integer step counts. The server also enforces a MAX_X_MM = 250 / MAX_Y_MM = 220 hard limit and returns HTTP 400 if the board is too large, with a message the web UI shows directly to the user.

# 8. Bugs and How I Fixed Them

This section is probably the most useful part of the report from a technical understanding perspective. These are the significant bugs I found and fixed during development. In each case I describe what was happening, why it was happening, and exactly what changed.

## 8.1 Ink Blobs at Every Corner

Symptom: every polygon corner on the drawn output had a small but visible ink blob, as if the pen had rested on the board for a moment before changing direction — because it had.

Root cause: the original protocol was strict stop-and-wait. The ESP32 sent a command, the PIC executed it from a standstill, the motor stopped completely, the PIC replied DONE, and only then did the ESP32 prepare and send the next command. Every direction change caused a full decelerate-halt-accelerate cycle, giving the pen a fraction of a second to sit stationary on the board and transfer extra ink via capillary action.

Fix: the protocol was redesigned so the ESP32 has the next frame already waiting in its UART transmit FIFO by the time the PIC's ACK arrives. Because 9600 baud transmission is fast relative to motor motion, the PIC receives the next command before the motor has fully stopped. From the motor's perspective, successive frames are continuous. The blobs disappeared completely.

## 8.2 24-Bit Carry Loss (the Hard One)

Symptom: plots of boards taller than about 163 mm would fail near the top. The Y carriage would stop 65,536 steps short of its target, crash into the mechanical end stop, and the plot would halt.

Root cause: 163 mm × 400 steps/mm = 65,200 steps, which is just under the 16-bit limit (65,535). Any Y coordinate above that required the middle byte of the 24-bit step count to roll over from 0xFF to 0x00. When that INCF executes, it sets the Zero flag — not the Carry flag — on the PIC. My carry propagation code only checked STATUS,C, so the carry into the high byte was silently dropped. Every Y coordinate above 163 mm was calculated as exactly 65,536 steps too small.

Fix: added a second check after every middle-byte INCF:

BTFSC   STATUS, C     ; carry from the previous ADDWF?

INCF    MULH, F       ;   yes → increment middle byte

BTFSC   STATUS, Z     ; did that INCF roll MULH to 0x00?

INCF    MULX, F       ;   yes → carry must propagate to high byte

This second BTFSC STATUS,Z / INCF pair is the bit I was missing everywhere. After adding it consistently to all 24-bit arithmetic blocks, the full coordinate range worked correctly.

## 8.3 Multiply-by-25 Was Multiply-by-5

Symptom: with the full-step firmware (25 steps/mm), every move was exactly 1/5 of the commanded distance. Commanding 100 mm produced 20 mm of travel.

Root cause: the shift-add multiplier for 25 (binary 11001) was missing the 8× shift term. The routine was computing 16× + 1× = 17×, and a second bug made it actually compute 4× + 1× = 5×. Fix: rewrote the routine from scratch against binary 11001 (bits 0, 3, 4), verified against Python for all inputs 0–65,535.

## 8.4 MUL8×16 High-Byte Corruption

Symptom: after the ×25 fix, moves over ~50 mm overshot by seemingly random amounts. Root cause: the carry-propagation in the multiplier used ADDLW to add 1 to W before adding to the high byte. ADDLW modifies W — so on the next iteration, W no longer held the multiplier byte as expected. Fix: replaced ADDLW/ADDWF with INCF (which never touches W), and added an explicit MOVF to reload the multiplier byte before each ADDWF.

## 8.5 Gerber Divisor (Drawings 100× Too Small)

Symptom: first real Gerber file drawn at 1/100th scale — the layout was recognisable but tiny. Root cause: hardcoded divisor of 10,000; the file used %FSLAX46Y46% which needs 1,000,000. Fix: runtime regex extraction of divisor = 10^(decimal field width) from the FS header.

## 8.6 Bresenham Direction Regression

Symptom: a brief firmware revision drew a mirror image along one axis. Root cause: tried to determine direction from the borrow flag of a 16-bit subtraction done as two 8-bit subtracts. Borrow does not propagate to the global carry bit correctly in this arrangement. Fix: reverted to explicit compare of target vs. current position before the loop, storing results in XDIR_F / YDIR_F flags. More readable and provably correct.

# 9. Testing and Validation

## 9.1 Proteus Simulation

Every significant firmware revision was run in Proteus before being flashed to real hardware. The simulation gives cycle-exact behaviour: the HD44780 virtual module shows the LCD output character-by-character, the virtual UART terminal lets you see exactly what bytes the PIC is receiving and transmitting, and logic analysers on PORTB show the actual step and direction pulses. About 80% of the bugs in Section 8 were found and fixed in simulation — not on hardware.

## 9.2 Step Calibration

Physical calibration: command exactly 100 mm in X and Y against a printed ruler. With 400 steps/mm and the T8 lead screw, the measured travel is within 0.1 mm of the commanded value. This confirms the firmware conversion constants are correct and the lead screw geometry matches the assumed 8 mm/rev lead.

## 9.3 End-to-End Plot Trials

Two regression test files: test_square.gbr (30×30 mm square with circular pads at corners — tests aperture expansion and corner motion) and test.gbr (complex trace pattern with diagonals, obrounds, overlapping polygons). Final validation involved drawing on copper-clad stock with resist-pen ink, etching in ferric chloride, inspecting under magnification, and testing trace continuity with a multimeter.

# 10. Final Specifications

## 10.1 Electrical

| Parameter | Value |
| --- | --- |
| Motor supply | 12 V DC |
| Logic supply | 5 V DC (regulated from 12 V) |
| ESP32 supply | 5 V (on-board linear regulator) |
| Stepper phase current | ~1.1 A per phase (A4988 VREF ≈ 0.60 V) |
| UART (ESP32 ↔ PIC) | 9,600 baud, 8N1 |
| WiFi | 2.4 GHz 802.11 b/g/n, WPA2-PSK |

## 10.2 Mechanical

| Parameter | Value |
| --- | --- |
| Work envelope | 250 mm (X) × 220 mm (Y) |
| Pen tip | 0.4 mm diameter |
| Raster fill step | 0.2 mm (half pen width) |
| Safety margin | 2 mm from origin on both axes |
| Homing | Micro-switch limits on PORTA,2 (X) and PORTA,3 (Y) |
| Frame material | 3D-printed PLA, 0.2 mm layers, 40% infill |

## 10.3 Motion and Resolution

| Parameter | Value |
| --- | --- |
| Step angle | 1.8° (200 full steps/revolution) |
| Lead-screw lead | 8 mm/revolution (T8, 4-start) |
| Micro-step resolution | 400 steps/mm at 1/16 (2.5 µm/step) |
| Step rate (calibrated) | 1,000 Hz |
| Feed rate (calibrated) | 2.5 mm/s linear (= 1000 Hz ÷ 400 steps/mm) |
| Max step count (X) | 100,000 steps (24-bit) |
| Max step count (Y) | 88,000 steps (24-bit) |

# 11. Conclusion

## 11.1 What Was Achieved

The machine works. You can take a Gerber file from KiCad or any other standard PCB tool, upload it through the browser, and the plotter draws it onto copper-clad stock without any further intervention. Line quality is good — corners are clean, diagonals are smooth, and pads are properly filled. All the design goals from Section 1.2 are met.

## 11.2 Things I Would Do Differently

The biggest thing I would change is the communication architecture between the ESP32 and PIC. Using a higher baud rate — 115,200 instead of 9,600 — would allow tighter ACK-response loops and potentially higher feed rates without any firmware complexity cost. I stayed at 9,600 because it was the working value and changing it mid-project felt risky, but in hindsight it is an easy upgrade.

I would also add a proper velocity profile — ramp up from zero, hold constant speed, ramp down before a stop. Right now the motor accelerates instantly from zero, which works but causes occasional missed steps on long diagonal moves that start at full speed. This requires a small look-ahead buffer and is doable in assembly.

# Appendix A — Complete Pin Assignments

## A.1 PIC16F877A

| Pin | Port | Dir | Signal | Connected to |
| --- | --- | --- | --- | --- |
| 33 | PORTB,0 | Out | X_STEP | A4988/DRV8825 STEP (X axis) |
| 34 | PORTB,1 | Out | X_DIR | A4988/DRV8825 DIR (X axis) |
| 35 | PORTB,2 | Out | Y_STEP | A4988/DRV8825 STEP (Y axis) |
| 36 | PORTB,3 | Out | Y_DIR | A4988/DRV8825 DIR (Y axis) |
| 4 | PORTA,2 | In | LIM_X | X limit switch, pull-down, active high |
| 5 | PORTA,3 | In | LIM_Y | Y limit switch, pull-down, active high |
| 25 | PORTC,6 | Out | UART TX | ESP32 GPIO16 (UART2 RX) |
| 26 | PORTC,7 | In | UART RX | ESP32 GPIO17 (UART2 TX) |
| 19–30 | PORTD,0–7 | Out | LCD D0–D7 | HD44780 data bus (8-bit mode) |
| 8 | PORTE,0 | Out | LCD E | HD44780 Enable |
| 9 | PORTE,1 | Out | LCD RW | HD44780 R/W (write only) |
| 10 | PORTE,2 | Out | LCD RS | HD44780 Register Select |

## A.2 ESP32 DevKit v1

| GPIO | Dir | Signal | Connected to |
| --- | --- | --- | --- |
| GPIO16 | In | UART2 RX | PIC PORTC,6 (TX) |
| GPIO17 | Out | UART2 TX | PIC PORTC,7 (RX) |
| GPIO23 | Out | Servo PWM | SG90 continuous-rotation servo signal |
| GND | Pwr | Ground | Common ground with PIC and driver modules |

# Appendix B — Firmware Constants

| Constant | Value | Where | What it controls |
| --- | --- | --- | --- |
| STEPS_PER_MM | 400 | PIC | Steps per mm at 1/16 micro-stepping |
| SPBRG | 25 | PIC | UART baud rate divisor (→ 9615 baud at 4 MHz) |
| STEP_DELAY | 1 ms | PIC | Inter-step gap; sets feed rate |
| MAX_STEPS | 100,000 | PIC | Hard coordinate clamp in firmware |
| SERVO_STOP | 95 | ESP32 | PWM value that stops the SG90 (calibrate per unit) |
| SERVO_CW_SPEED | 110 | ESP32 | Pen-UP spin speed |
| SERVO_CCW_SPEED | 70 | ESP32 | Pen-DOWN spin speed |
| SERVO_MOVE_MS | 400 | ESP32 | Time to spin between pen states (ms) |
| ACK_TIMEOUT_MS | 60,000 | ESP32 | Max wait for PIC ACK before error |
| FETCH_CHUNK_SIZE | 4,096 | ESP32 | Bytes per /fetch request (~270 frames) |
| MAX_X_MM | 250 | Flask | Work envelope X limit |
| MAX_Y_MM | 220 | Flask | Work envelope Y limit |
| FILL_STEP | 0.2 | Flask | Pad raster step in mm |
| MIRROR_X | True | Flask | Mirror X for copper-side-up plotting |

# Appendix C — End-to-End Frame Example

This traces one motion command from Gerber file all the way to physical motor pulses, to show how all the layers connect.

### Gerber input

%FSLAX46Y46*%    ; divisor = 1,000,000

%MOMM*%           ; metric

X100000Y050000D01*  ; raw coords: X=0.10 mm, Y=0.05 mm, pen down

### Flask processing

x_mm = 100000 / 1,000,000 = 0.10 mm

y_mm =  50000 / 1,000,000 = 0.05 mm

Add 2mm margin: x = 2.10 mm, y = 2.05 mm

Convert to steps: x = 2.10 × 400 = 840, y = 2.05 × 400 = 820

pen = '1' (D01 = draw)

Output frame: I0008400008201\n

### ESP32 transmission

Serial2.write("I0008400008201\n", 15);

// 15 bytes at 9600 baud → transmission takes ~15.6 ms

// ESP32 transitions to WAITING_ACK

### PIC reception and motion

RECEIVE_LINE: BUF = ['I','0','0','0','8','4','0','0','0','8','2','0','1','\n']

Validation: start='I' ✓  digits ✓  pen='1' ✓

XTGT = 840, YTGT = 820, PENVAL = '1' (pen down)

EXECUTE_MOTION:

Assume previous position was (0, 0)

DX = 840, DY = 820 → dominant axis = X (840 > 820)

Bresenham runs 840 iterations:

X steps every iteration (840 total)

Y steps when error underflows (820 total)

At 1000 Hz: ~840 ms of motion

SEND_ACK: TX → '1' (pen state echo), 'A' (acknowledge)

### ACK received

ESP32 sees '1' → updates servo state, sees 'A' → returns to PLOT_CHUNK and sends the next frame. Total round-trip dominated by motion time. UART overhead is under 2% of total command time.